AV01 App

Application Layer Reference and Functional Guide

This document describes the AV01 application layer, including build variants, execution flow, telemetry, event handling, barometric processing, apogee detection, and SD raw logging. The text is intentionally explanatory so it can be read by developers, integrators, and reviewers who are not deeply familiar with embedded flight software.

Contenido

1. Introduction	2

2. Intended audience and reading guide	2

3. High-level architecture	2

4. Build variants	3

5. Main execution flow	3

6. Public interface	3

7. Core data structures	4

8. Scheduler module (app_tasks)	4

9. Barometric processing path	4

10. Atmosphere helper (atmos)	5

11. Filter helper (filters)	5

12. Apogee detector	5

13. Logging subsystem	6

14. Event queue and health monitoring	6

15. Flight build behavior in detail	6

16. Diagnostic build behavior in detail	7

17. Configuration reference	7

18. Assumptions, limitations, and integration notes	7

19. File-by-file responsibility summary	8

20. Conclusion	8

# 1. Introduction

The AV01 app is the top-level coordination layer that turns several lower-level modules into a usable flight application. The app itself does not directly talk to raw peripherals in a complex way; instead, it orchestrates a small set of services: a periodic scheduler, a barometric acquisition path, an apogee detector, an event queue, and an SD logging subsystem. In practice, this means the app is responsible for deciding what is executed, how often it is executed, and how the resulting data is exposed to the rest of the firmware.

The code base is intentionally split into small modules. This makes the system easier to reason about, easier to test, and easier to configure for different operating modes. Two build variants are supported: a diagnostic build intended for bench validation and bring-up, and a flight build intended for autonomous operation with logging and apogee detection enabled.

# 2. Intended audience and reading guide

• Firmware developers can use this document as a map of the application architecture and public interfaces.

• Systems or test engineers can use it to understand the runtime behavior without reading every source file in detail.

• Reviewers with limited embedded background can focus on the execution flow, state machine description, and configuration tables first, and then move to the API details later.

# 3. High-level architecture

At runtime, the app behaves like a cooperative executive. A millisecond tick provided by the OS abstraction layer is used as the common time base. Periodic tasks are declared with a period, a next execution time, and a callback function. The main loop calls the scheduler, and the scheduler invokes whichever tasks are due. Because this is cooperative rather than preemptive, each task is expected to finish quickly and avoid blocking operations whenever possible.

| Layer or concept | Implementation | Role in the application |
| --- | --- | --- |
| Application entry points | app_init(), app_loop(), app_get_telemetry() | Initialize the whole subsystem, run periodic work, and expose a current snapshot. |
| Task scheduler | app_tasks.* | Provides a minimal periodic dispatcher based on period_ms and next_ms fields. |
| Barometric processing | baro.*, atmos.*, filters.* | Reads pressure/temperature, computes a relative altitude baseline, filters altitude, and estimates vertical speed. |
| Flight logic | apogee.* | Implements a small state machine that declares apogee after a sustained negative vertical speed. |
| Data persistence | log.* | Packages telemetry and events into framed binary records and forwards them to the raw SD logging backend. |
| Cross-module data | telemetry_t and app_event_t | Carries current measurements and significant events across the system. |

# 4. Build variants

The build selection is centralized in app_cfg.h. If neither APP_DIAG_BUILD nor APP_FLIGHT_BUILD is explicitly defined, the code defaults to the flight build. This default matters because the enabled modules are different in each mode.

| Build | Enabled behavior | Typical use |
| --- | --- | --- |
| APP_DIAG_BUILD | Barometer enabled, UART diagnostic output enabled, SD logging disabled by default, apogee disabled by default | Bench bring-up, sensor validation, quick visibility over serial output |
| APP_FLIGHT_BUILD | Barometer enabled, SD logging enabled, apogee enabled, UART diagnostic output disabled | Autonomous flight operation with event detection and persistent data capture |

This separation is useful because bench diagnostics often prioritise readability and operator feedback, while flight software prioritises deterministic behavior and storing data for later analysis. The code therefore keeps the same core modules but changes which tasks are instantiated at start-up.

# 5. Main execution flow

• app_init() clears the global application context, initializes communication wrappers, starts the selected modules, builds the periodic task list, and seeds each task with its first release time.

• app_loop() reads the current tick count, updates the telemetry timestamp, and asks the task scheduler to run every task that is due at that moment.

• Each task updates one part of the application state: sensor acquisition, apogee logic, logging, flushing, health supervision, or diagnostic printing depending on the selected build.

• The telemetry snapshot is stored in the global context and can be read at any time through app_get_telemetry().

# 6. Public interface

| Function | Description |
| --- | --- |
| app_init(void) | Performs one-time initialization of the application context and the enabled modules. It must be called before app_loop(). |
| app_loop(void) | Executes the periodic application work. It is expected to be called repeatedly by the higher-level firmware loop. |
| app_get_telemetry(void) | Returns a const pointer to the current telemetry snapshot so that other software layers can inspect the latest values. |

# 7. Core data structures

Two shared data structures are particularly important: telemetry_t and app_event_t. Together they represent the current state of the vehicle and the noteworthy transitions that should not be lost.

| telemetry_t field | Meaning |
| --- | --- |
| t_ms | Current application time in milliseconds. |
| p_centi_mbar | Measured pressure expressed in centi-mbar. This keeps storage compact while avoiding floating-point transport. |
| t_centi_c | Measured temperature in centi-degrees Celsius. |
| alt_mm | Relative altitude in millimetres, computed against a start-up pressure baseline. |
| vel_mm_s | Estimated vertical speed in millimetres per second. |
| flags | Bit field describing health and status, including barometer availability, SD/log readiness, apogee detection, and dropped logging bytes. |
| state | Current high-level flight state encoded from app_state_t. |
| dropped_bytes | Number of bytes that the logging system could not enqueue or preserve. |

| Event | When it is emitted |
| --- | --- |
| EVT_APOGEE_DETECTED | Generated once when the apogee state machine transitions into APPOGEE-latched state. |
| EVT_BARO_FAIL | Generated when barometric acquisition was previously healthy and becomes unhealthy. |
| EVT_SD_FAIL | Generated when SD/logging availability is lost or initialization fails. |

# 8. Scheduler module (app_tasks)

The scheduler is intentionally minimal. A task is a structure containing a period in milliseconds, the next absolute tick at which it should execute, a callback pointer, and an optional context pointer. tasks_init() simply converts each relative period into an initial absolute deadline. tasks_run_due() scans the task array and executes any task whose deadline has passed or been reached.

An important detail is that the scheduler advances next_ms by period_ms after each release instead of resetting it from the current time. This helps preserve average timing even if a particular loop iteration arrives slightly late. It also means the task model is deterministic and easy to inspect: every task is periodic, and there is no dynamic insertion or removal during normal execution.

# 9. Barometric processing path

The barometric path is spread across three small modules because each module performs a clearly defined job. baro.* owns the measurement state and the temporal logic, atmos.* converts pressure into relative altitude, and filters.* applies a simple first-order IIR filter in fixed-point Q15 arithmetic.

• baro_init() resets the barometer context, initializes the MS5837 driver, and starts the sensor with an oversampling setting of OSR 4096.

• baro_update() reads the latest pressure and temperature values. During an initial warm-up period, it accumulates pressure samples in order to compute a baseline p0.

• Before the baseline is ready, the sensor is still considered operational, but altitude and vertical speed are deliberately reported as zero to avoid misleading transients.

• Once the warm-up duration has elapsed, the average baseline pressure is frozen. Relative altitude is then computed from the ratio between current pressure and baseline pressure using a standard atmosphere equation.

• The resulting altitude is filtered. Vertical speed is obtained by differentiating filtered altitude with respect to elapsed time and then filtering that derivative again.

A small clamp is applied to the elapsed time used in the derivative calculation. This prevents pathological values if the scheduler is called too quickly or too slowly. As a result, the velocity estimate remains numerically stable even if the main loop timing is imperfect.

# 10. Atmosphere helper (atmos)

The atmos_alt_mm_from_p_centi_mbar() function converts a pressure reading into relative altitude in millimetres using a simplified ISA-style barometric formula. The function first converts the fixed-point centi-mbar inputs into floating-point mbar values, computes the pressure ratio p/p0, and then applies the standard pressure-altitude relationship. Because the application only needs relative altitude referenced to its own baseline, this method is appropriate and simple.

# 11. Filter helper (filters)

The filtering helper exposes one function: iir1_q15(). This implements a first-order low-pass response in fixed-point form using the update y += alpha * (x - y). The alpha coefficient is represented in Q15 format, where 32768 corresponds approximately to 1.0. This avoids transporting floating-point coefficients throughout the higher-level code while still providing smooth and cheap filtering.

# 12. Apogee detector

The apogee detector is intentionally conservative and easy to understand. It is modelled as a three-state machine: IDLE, ASCENT, and APOGEE. On start-up, the detector stays in IDLE. It only arms into ASCENT once both altitude and upward velocity exceed configured thresholds. This prevents the detector from reacting to bench noise or small motions near the ground.

Once armed, the detector watches for a sustained negative vertical speed. A single negative sample is not enough. Instead, the condition must remain below APOGEE_NEG_VEL_DETECT_MM_S long enough to accumulate APOGEE_HOLD_MS. When that hold time is reached, the state changes to APOGEE and remains latched. The one-shot event flag out_apogee_event is asserted only at the transition edge.

| Threshold | Purpose |
| --- | --- |
| APOGEE_MIN_ALT_ARM_MM | Minimum altitude before the state machine can move out of IDLE. |
| APOGEE_ASCENT_VEL_ARM_MM_S | Minimum positive velocity required to confirm that ascent is real. |
| APOGEE_NEG_VEL_DETECT_MM_S | Negative velocity threshold used to infer that the peak has been passed. |
| APOGEE_HOLD_MS | Persistence time used to reject short disturbances or noise around zero speed. |
| APOGEE_FILTER_ALPHA_Q15 | Filter coefficient shared by altitude and velocity estimation in the barometric path. |

# 13. Logging subsystem

The logging subsystem wraps a lower-level raw SD logger. From the application point of view, its responsibilities are to initialise the SD backend, frame data, append telemetry or event records, poll the backend to flush pending pages, and expose the count of dropped bytes.

Each record begins with a small packed header containing a type, a version number, and the payload size. This makes the log stream self-describing enough to parse offline. Telemetry frames carry a full telemetry_t snapshot, and event frames carry one app_event_t object. Timestamps are also forwarded to the backend as ordering metadata.

| Logging parameter | Explanation |
| --- | --- |
| LOG_PAGE_BYTES | Size of one raw log page buffered before physical media write. |
| LOG_NUM_BUFFERS | Number of RAM buffers available for the logging backend. |
| LOG_USE_CRC | Enables frame/page integrity support in the backend. |
| LOG_WRAP | Allows writing to wrap around the configured logging region. |
| LOG_USE_PREERASE | Requests pre-erase optimisation when supported. |
| LOG_META_PERIOD_PAGES | Controls how often metadata is refreshed. |
| LOG_META_LBA / LOG_DATA_LBA / LOG_END_LBA | Define the raw card area reserved for metadata and log payloads. |

# 14. Event queue and health monitoring

Both app variants use a small ring-buffer event queue. The queue stores event objects in a fixed-size array and tracks head, tail, and count indices. Pushing fails cleanly when the queue is full; the code does not block or overwrite older events. This choice favours predictable behavior over aggressive retention.

In the flight build, a dedicated health task watches whether the barometer and logging subsystem remain healthy. It compares the current status against the previous status and emits events only on falling edges. In other words, the system records the moment a resource fails, not a repetitive failure every cycle. This is a good pattern for event logs because it keeps them compact and more informative.

# 15. Flight build behavior in detail

• The barometer task runs periodically and updates pressure, temperature, altitude, and vertical speed.

• The apogee task consumes the latest altitude and velocity estimate, updates the current app_state_t value, and emits an apogee event when the detection condition is met.

• The log task captures the latest telemetry snapshot and, if available, one queued event per execution.

• The flush task services the logging backend so buffered pages are written to the SD card.

• The health task propagates SD/log status into telemetry flags and emits resource-failure events when a previously good resource becomes bad.

This ordering of responsibilities keeps the design simple: measurement first, decision making second, recording third, maintenance in parallel. Because each task uses the same shared telemetry snapshot, the system behaves like a continuously refreshed data pipeline.

# 16. Diagnostic build behavior in detail

The diagnostic build keeps the same overall structure but replaces autonomous flight functions with operator-facing output. The barometer task updates the telemetry values, and the print task periodically formats a human-readable UART line that includes time, barometer status, baseline status, pressure, temperature, altitude, velocity, and flags. If an event is available, the print task also emits a second line naming the event.

This is especially useful during integration because the user can verify baseline readiness, observe whether pressure is updating, and see failures immediately without needing to extract an SD card or decode a binary log.

# 17. Configuration reference

| Constant | Default value | Meaning |
| --- | --- | --- |
| APP_BARO_HZ | 50 Hz | Rate of barometric updates. |
| APP_APOGEE_HZ | 50 Hz | Rate of apogee evaluation in flight build. |
| APP_LOG_HZ | 50 Hz | Rate at which telemetry and queued events are appended to the log. |
| APP_FLUSH_HZ | 200 Hz | Rate at which the logging backend is polled for pending writes. |
| APP_HEALTH_HZ | 10 Hz | Rate of health supervision in flight build. |
| APP_DIAG_PRINT_HZ | 5 Hz | UART print rate in diagnostic build. |
| APP_EVENT_Q_LEN | 8 entries | Depth of the event ring buffer. |
| BARO_BASELINE_WARMUP_MS | 1500 ms | Warm-up window used to estimate baseline pressure. |

# 18. Assumptions, limitations, and integration notes

• The app assumes osal_ticks_ms() is monotonic and reasonably stable. If the system tick is unreliable, scheduler timing and velocity estimation will degrade.

• Altitude is relative to the pressure measured during the start-up warm-up interval. It is not an absolute altitude above sea level.

• The quality of apogee detection depends on the quality of barometric readings and on correct threshold tuning for the expected flight profile.

• The logging path relies on lower-level SD and raw logging modules that are not described in full here, but their application-facing contract is clear through logsys_* wrappers.

• The application keeps a single global context instance, which is simple and practical for embedded systems but implies the code is not designed for multiple concurrent app instances.

# 19. File-by-file responsibility summary

| File | Responsibility |
| --- | --- |
| app.h | Public entry points of the application layer. |
| app_cfg.h | Central build-time feature selection and numeric configuration constants. |
| app_types.h | Application states, event identifiers, event payload structure, and telemetry snapshot structure. |
| app_tasks.h / app_tasks.c | Small cooperative periodic scheduler. |
| app_flight.c | Flight-oriented top-level integration including barometer, apogee, logging, and health monitoring. |
| app_diag.c | Diagnostic-oriented top-level integration including barometer, optional logging, and UART printing. |
| baro.h / baro.c | Barometer state and update path. |
| atmos.h / atmos.c | Pressure-to-relative-altitude conversion. |
| filters.h / filters.c | Fixed-point first-order low-pass helper. |
| apogee.h / apogee.c | Flight state machine used for apogee declaration. |
| log.h / log.c | Application-level wrapper around raw SD logging. |

# 20. Conclusion

The AV01 application layer is compact but well structured. Its main value lies not in complex algorithms, but in how it assembles simple modules into a predictable flight-ready workflow. The scheduler provides timing, the barometric path produces navigation-like signals, the apogee detector turns those signals into a meaningful flight event, and the logging system preserves both continuous telemetry and exceptional conditions. Because responsibilities are separated cleanly, the application is approachable for new readers and straightforward to extend.